TECHNICAL SPECIFICATION

Manufacturer: 			Safelab Systems Ltd

Address:	Airone Building, 8 Beaufighter Road, Weston-Super-Mare, BS24 8EE

Telephone:	+44 (0)1934 421340

E-mail:				enquiries@safelab.co.uk

Website: 			www.safelab.co.uk

Supplier Contact:			Adrian Wyles – email: adrian@safelab.co.uk tel: +44 (0)7398 288292

Product description:		Lockable Service Docking station

Range:	Docking station for use with Airone RS mobile filtration & Airone DS Semi mobile ducted fume cupboards

Product Code:			Docking station

Application: 	For use in Science Laboratories with Airone RS mobile filtration & Airone DS Semi mobile ducted fume cupboards. Designed for use in school laboratories. Allows quick and easy connection of services utilising bayonet connections. The docking station stops unauthorised access to mains service supplies, enabling water, waste, and gas to be securely locked away when not in use.

Standards:	Designed to comply with CLEAPSS G9 Fume cupboards in Schools.

Guarantee:	5 years warranty when Safelab carry out the annual service.

Installation:	Can be recess mounted or surface mounted – optional wall spacer is also available

External dimensions:		200mm wide x 110mm deep x 277mm high – excluding mounting bracket

External carcase: 	Manufactured from 16swg mild steel and painted in epoxy powder coating

Lockable hinged access door:	Manufactured from 16swg mild steel and painted in epoxy powder coating – reversible – left or right-handed

Tether hook:	To accept gas safety stainless steel cable on the Airone RS & Airone DS

Weight:				4kg

Services:

Gas fitting:	1/8” Quick release fitting - 1/8” BSPT to 10mm compression bulkhead fitting

Water fitting:	1/4” Quick release fitting – 1/4” BSPT to 10mm compression bulkhead fitting

Waste:	1 1/2” BSP waste pipe coupler & Blanking plug.

Electrical sockets:	Fascia mounted Single 13Amp outlet